Description of cases

Date: 21.08.21 Karolina

Decane + ice.

Weight of big plastic tank: 0.37 kg (2 pieces)

Weight of small plastic tank: 0.152 kg (2 pieces)

Weight of decane: 19.856 kg

Expected loading of ice: 7% vol

Required weight of ice: 1.830 kg

Crashed weight of ice: 1.837 kg

Loaded weight of ice: 1.776 kg

Underload of ice: 0.061 kg

Targeted temperature: -1 C

Targeted flowrate: 400 kg/h

Underload of decane: 0.224 kg

Real weight of decane in the loop: 19.632 kg

Real ice concentration: 6.9% vol.

Pump frequency: 11.3 Hz (11h39m), 11.4 (11h43m), 11.6 (11h44m), 16.2 (11.51) 16.8 (11.57) 15.2 (12.12)

The result of the test: Blockage in 70 minutes. During the test, a significant decrease in flow rate was observed several times due to the partial blockage of the orifice hole. During the first series of partial blockages, the flow was forcedly restored by increasing the pump frequency. This led to the grinding of the deposit and restoring the flow rate at the pump frequencies close to initials. In the second series of blockages, the flow rate was not recovered forcibly. When the flow rate decreased, the pump continued to operate at the same frequency without changing. As a result, in the second series, slight decreases in flow were observed, with subsequent restoration to the previous value (without changing the pump frequency!). After that, there was a complete blocking of the orifice. Attempts to increase the frequency of the pump after blocking resulted in a seal on the plug. The flow through the orifice`s hole has not been restored.

Date: 01.09.21 Karolina

Decane + ice.

Weight of big plastic tank: 0.37 kg (2 pieces)

Weight of small plastic tank: 0.152 kg (2 pieces)

Weight of decane: 19.366 kg

Expected loading of ice: 6.5% vol

Required weight of ice: 1.653 kg

Crashed weight of ice: 1.714 kg

Loaded weight of ice: 1.643 kg

Underload of ice: 0.071 kg

Targeted temperature: -1 C

Targeted flowrate: 400 kg/h

Underload of decane: 0.173 kg

Real weight of decane in the loop: 19.193 kg

Real ice concentration: 6.5% vol.

Pump frequency:

The result of the test: A large deposit was obtained on the bottom of the horizontal pipe up to the orifice during the main test. No blockage or signs of blockage were found. Visually, a decrease in the concentration of particles in the stream is noticeable. After the main test, the loop was purged, and a test was carried out, as a result of which a partial blockage of the hole was detected. However, the result cannot be taken into account due to the lower concentration of particles: as a result of the purge, it was not possible to get rid of the entire mass of the particle deposit. The reason for partial blockage after the purge can be ice freezing from the water formed in the loop during the first part of the experiment and partial melting of particles (insignificant amount).

Concentration changing: 1 – 3.9% (-003 min), 2 – 5.7% (006 min), 3 – 4% (012 min), 4 – 3.8% (018 min), 5 – 2.5% (025 min), 6 – (033 min)

Date: 23.09.21 Karolina

Decane + ice.

Weight of big plastic tank: 0.37 kg (2 pieces)

Weight of small plastic tank: 0.152 kg (2 pieces)

Weight of decane: 18.458 kg

Expected loading of ice: 10% vol

Required weight of ice: 2.519 kg

Crashed weight of ice: 2.630 kg

Loaded weight of ice:  kg

Underload of ice:  kg

Targeted temperature: -1 C

Targeted flowrate: Differential pressure test!

Underload of decane:  kg

Real weight of decane in the loop:  kg

Real ice concentration: % vol.

Pump frequency:

The result of the test: The differential pressure was measured for the flow rates 2000-1000 kg/h. At decreasing flow rate from 1000 to 800 the fast blockage was got.

Concentration changing: The samples were taken at 2000 kg/h: 6.4%, 9.1%, 8.5%

Date: 18.03.22 Karolina

Decane + ice.

Weight of big plastic tank: 0.37 kg (2 pieces)

Weight of small plastic tank: 0.152 kg (2 pieces)

Weight of decane: 17.441 kg

Expected loading of ice: 5% vol

Required weight of ice: 1.126 kg

Crashed weight of ice: 1.192 kg

Loaded weight of ice: 1.132 kg

Underload of ice: 0.022 kg

Targeted temperature: -1 C

Targeted flowrate: 900 kg/h

Underload of decane: 0.111 kg

Real weight of decane in the loop: 17.33 kg

Real ice concentration: 5.3 % vol.

Pump frequency:

The result of the test: The video has been started at 11:05. Slight reducing og the flow rate (00 -> 830 kg/h)/ the sedimentation is around the glass pipe before the orifice, and in the top part behind the orifice.

The second part of the test was done with higher temperature at the test section inlet. The video has been started at 17:32. In a result, the partial blockage of the orifice and, then, the full blockage of the loop behind the test section. Due to high temperature, the sediment behind the orifice was removed by the flow.

Concentration changing:

11:22 -

11:52 – 5.3%

12:52 – 5.0%

14:58 – 4.4%

17:00 – 3.7%

Date: 24.01.23 Karolina

Ice concentration: 15.1%

Temperature at TS inlet: –1.2 °C (in the tank)

Test result: Loop blockage

Blockages in the video:

06:40 – washer blockage

15:04 – washer blockage

Run timecodes:

15:11:12 – 15:16:10 – loop blockage

15:19:56 – 15:24:32 – washer blockage

If you need the full set of translations merged into one document, I can prepare it.

Date: 07.09.22 Karolina

Ice concentration: 15%

TS inlet temperature: –0.8 °C (–1.0 °C in the tank)

Test result:

Blockages on video:

39:34 – blockage of the test section

Start time codes:

13:15:49 – 13:44:06 – blockage of the test section

Notes:

The video starts at 13:05. Blockage of the test section. Initially, the pump frequency was set to 10 Hz, which practically did not lead to any deposit formation in the test section. Then the pump frequency was reduced to 8 Hz. As a result, deposit formation was observed near the orifice simultaneously with deposit formation in the riser tube. This led to a gradual decrease in flow rate and, ultimately, to blockage of the test section.

Date: 21.06.22 Karolina

Ice concentration: 15%

Temperature at TS inlet: –2 °C

Test result: Three runs were conducted: blockage of the loop in the first run, blockage of the test section in the second run, and no blockage in the third run.

Blockages in the video:

Run timecodes:

13:57:15 – 13:59:21

14:01:48 – 14:06:57

14:27:16 – 14:31:41

Date: 29.06.22

Ice concentration: 15%

Temperature at TS inlet: –1 °C

Test result: Video starts at . Four runs were conducted; blockage was achieved in the second, third, and fourth runs.

Blockages in the video:

57:30 – test section

1:12:37 – test section

1:36:38 – test section

Run timecodes:

14:33:05 – 15:14:26

15:20:20 – 15:28:59

15:33:24 – 15:44:06

15:49:03 – 16:08:07

Clearly pronounced blockages of the test section. A slight increase in tank temperature led to a smaller amplitude of the pressure spike in the test section.

Date: 29.06.22 Karolina

Ice concentration: 15%

Temperature at TS inlet: –1 °C

Test result: Video starts at . Four runs were conducted; blockage was achieved in the second, third, and fourth runs.

Blockages in the video:

57:30 – test section

1:12:37 – test section

1:36:38 – test section

Run timecodes:

14:33:05 – 15:14:26

15:20:20 – 15:28:59

15:33:24 – 15:44:06

15:49:03 – 16:08:07

Clearly pronounced blockages of the test section. A slight increase in tank temperature led to a smaller amplitude of the pressure spike in the test section.

Date: 30.06.22 Karolina
Ice concentration: 15%
Temperature at TS inlet: –1 °C
Test result: Three runs were conducted; blockage was achieved only in the third run. Also, before the start of the second and third runs, a small non-removable sediment remained on the glass tube.
Blockage on video at 3:15:45 (third run). Loop blockage was achieved.

Run timecodes:
13:53:03 – 15:02:50
15:08:37 – 16:08:03
16:15:39 – 17:03:21 (loop blockage)

Date: 05.07.22 Karolina
Ice concentration: 15%
Temperature at TS inlet: –2 °C
Test result: ….

Run timecodes:
15:37:26 – 16:17:41
16:20:34 – 16:43:20